IEEE 1451.4 Template and TDL Items

Registration Request Process Overview

Charles H. Jones

16 September 2004

# Overview

The implementation of the IEEE 1451.4 is dependent on templates and the associated template description language (TDL).  General descriptions of these can be found in the standard itself or in tutorials available on the website.  This document assumes the reader is familiar with what these items are.

A mechanism for manufacturers to request formal registration of new TDL items has been established for various reasons including:

- Templates and TDL items are critical to the implementation of the standard.

- The working group of the IEEE 1451.4 standard recognized that they could not possibly identify all necessary TDL items that any manufacturer would want to use.

- A central location for collection, publication, and dissemination of these items is necessary in order to maintain standard terminology and usage of these items.

The general process for registering TDL items is described on the website and consists mainly of filling out the appropriate request form (contained on the website) and submitting appropriate documentation.  This document identifies specifically what the appropriate documentation is for each of the TDL items.

In addition to templates, the term ‘TDL item’ generically refers to the keywords that define the TDL.  The intent is that, as manufacturers develop their own templates and discover they need some new functionality in the TDL, that they may request official recognition of their templates and TDL items.  By placing these items on the official website, they can be publicly disseminated. However, some classes of keywords may be added to through this process while other classes will have to wait until a formal revision of the standard is released.

The TDL items that may be published through this request process are:

- Manufacturer defined templates (see clause 7.2.1).

- Property commands (see clause 7.4 and Annex B.)

- Enumerated types (see clause 7.4.5.4 and Annex A.3.)

- Physical units (see clause 7.4.7 and Annex A.4.)

TDL items that may not be added to or published through this request process include:

- IEEE 1451 standard templates (manufacturer ID 0, see clause 7.2.1 and Annex A.)

- Identification commands (see clause 7.2.)

- Control commands (see clause 7.3.)

- Access levels (see clause 7.4.4.)

- Datatypes (see clause 7.4.5.)

- Format parameters (see clause 7.4.6.)

- Subproperties (see clause 7.4.9.)

People that believe there are desirable additions to these items which can not be added to through this process are encouraged to document them and present them to the working group when the standard is reviewed.

Note well that requests for templates and TDL items to be registered should not be made until those items have been thoroughly tested and, preferably, have gone through extensive use in the field.

Note well that registering a template or TDL item means that the manufacturer agrees to full public release of that item and associated documentation.

Mechanisms exist for each of the items that can be registered to be implemented and used without formal registration.  (In fact, this is the fundamental reason why the other items can not be registered.)  More specifically:

- The distinction between a manufacturer defined template and an IEEE standard template is that IEEE standard templates use manufacturer ID 0 in the TEMPLATE.  Thus, any manufacturer may create their own templates using their own manufacturer ID in the TEMPLATE command.  At the time that the the standard is formally revised, all registered templates will be thoroughly reviewed and considered for inclusion as standard templates

- Manufacturers may implement property commands through the use of %MDEF (see clause 7.4.10.1.)  That is, manufacturer defined properties can be created by using the MDEF prefix.  However, the use of this approach implies that not all applications parsing a template will recognize the property.  Thus, it is expected that when a manufacturer creates a new property, it is first implemented and used in the field for a while using the MDEF prefix.  Then, after any potential bugs are worked out, the manufacturer requests registration of the property.  At which time the MDEF prefix is removed.

- Any enumerated type or physical unit that is used in a template must be defined in that template using the appropriate command.  As such, there is no absolute need for any list of these items outside the individual templates.  However, it is desirable to establish standard lists of these items to encourage consistency and interoperability.  Thus, there is an initial list of these items in Annex A and manufacturers are encouraged to add to these lists.

For all of these, it is strongly recommended that, when creating a manufacturer defined template or item, that the manufacturer also produce the documentation described below in anticipation of registering the template or item.  This could also serve as documentation for customers on the use of these items as well.

When requesting to register a template or TDL item, all documentation must be submitted in electronic form.

# Registering a Template

These are the items and formats required when submitting a request to register a template.

## Documentation Required

- The template.

- A summary table of the template similar in format to those used in Annex A.  See Section 2.3 below for specifics.

## Template Format Requirements

- The electronic version of the template must be an ASCII text file with extension .tdl. This file must contain the correct Validation_Keycode.

- The template must conform to the syntax described in Clause 7 of the standard.  This can be verified by running the template through the TDL syntax checker provided in the TDL Startup Kit.

- All TDL items used in the template must be formally registered and defined.  The use of manufacturer defined properties using %MDEF is not allowed in registered templates.

- A description of the template and its use must be provided as comments at the top of the template.  This description should be in complete sentences and grammatically correct.

- The Manufacturer ID in the Template command must be a registered Manufacturer ID and must be the Manufacturer ID of the manufacturer submitting the request.

- The Template ID in the Template command must be unique among registered templates with the same Manufacturer ID.

- The Template command must include a title.

## Template Summary Table Format

Summary tables for templates provide a breakdown of all the fields defined in a template.  The following table format must be used.  There are examples in Annex A for each of the IEEE standard templates.  Note that the select column allows breakdown of different cases possible through the use of the SELECTCASE command.  The bottom row includes a calculation of the range of how many bits are required for the template.  Note that the syntax checker calculates this information.

| Function | Select | Property/ Command | Description | Access | Bits | Data type (and range) | Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID |  | TEMPLATE | Template ID |  | 8 | Integer (value = 35) |  |
| … | … | … | … | … | … | … | …. |
| Total bits required for TEDS (range): |  |  |  |  | XXX to XXX bits |  |  |

# Registering a Property Command

## Documentation Required

- A text file containing syntactical examples.  If the property uses physical units or enumerations, then the associated commands for these must be included.

- A full description of the property.  This description should have the same form as the descriptions provided for properties in Annex B.  The detail of the description must allow any person to fully use the property.  If appropriate, equations, graphs and example uses should be provided.

## Property Command Requirements

- The property must conform to the syntax of properties as described in Clause 7.4.  This can be verified by placing the syntactical examples inside a template and using the syntax checker.

- The name of the property must not duplicate any previously registered property name.

- The property must not be redundant in functionality with other approved properties.  If the requested property is similar to another property, then a description of how they differ must be included.

# Registering Enumerations and Physical Units

These are the simplest to register.  All that is needed is to provide the complete ENUMERATE or PHYSICAL_UNIT command.  A PHYSICAL_UNIT command should include a comment at the end describing the physical unit.  The name of the enumeration or physical unit must not be the same as a previously registered enumeration or physical unit. The list of enumerations in the requested enumeration command should not be the same as the list for an already registered enumeration.  The syntax can be verified by putting the commands in a template and using the syntax checker.  For examples, see Annex A3 and A.4.